Development of a Conceptual Storage Ring Structure for the Study of the Electric Dipole Moment of Light Nuclei

P. I. Palamarchuka (https://orcid.org/0009-0005-0248-3813)a,c , S. D. Kolokolchikova,b, A. E. Aksentyeva,b,c, A. A. Melnikova,b,d, Yu. V. Senicheva,b

aInstitute for Nuclear Research RAS, Moscow, Russia,

bMoscow Institute of Physics and Technology, Dolgoprudny, Russia,

cMoscow Engineering Physics Institute, Moscow, Russia,

dInstitute for Theoretical Physics L.D. Landau, Chernogolovka, Russia

*e-mail: patri3@inbox.lv

Abstract — This work presents the general concept of a storage ring structure for the investigation of the electric dipole moment (EDM) of the proton and deuteron. The main features of frozen-spin and quasi-frozen-spin structures, designed to suppress spin rotation due to the magnetic dipole moment (MDM), are described. For the implementation of the proposed methods, the dependence of the fields in spin-compensating elements is determined.

Keywords: frozen spin method, quasi-frozen spin method, electric dipole moment, storage ring

INTRODUCTION

One of the conditions for the predominance of matter over antimatter in the observable Universe is the violation of T-symmetry, the presumed source of which may be the existence of an electric dipole moment (EDM) of elementary particles [1, 2]. A cyclic accelerator operating in a storage ring mode serves as a tool for measuring the EDM of charged particles.

The investigation of the EDM is carried out through its effect on the spin dynamics of particles. To compensate for the precession arising from the presence of a magnetic dipole moment (MDM), either the frozen-spin or quasi-frozen-spin methods are employed. The optimal application of these techniques varies for different particles. In order to determine the principal design of a structure capable of measuring both the proton and deuteron EDM, a comparison of existing methods was performed in terms of their efficiency.

.

SPIN DYNAMICS

According to the Ehrenfest theorem, in quantum mechanics the expectation values of a particle’s coordinates and momenta, as well as the forces acting on it, are related by equations analogous to the corresponding equations of classical mechanics. For spin, the evolution in external fields is described by the Thomas-Bargmann-Michel-Telegdi (T-BMT) equation [3]:

dSdt&=ΩMDM+ΩEDM×S,ΩMDM&=-qmγγG+1B⊥+G+1B∥-γG+γγ+1β×Ec,   ΩEDM&=-qη2mβ×B+Ec-γγ+1βc(β∙E) ,  G=g-22,(1)

where ΩMDM,ΩEDM — angular velocities of spin precession induced by the MDM and EDM, respectively; q,m,G are the charge, mass, and anomalous magnetic moment, β — is the normalized velocity; γ — Lorentz factor; d=ηq2mcs — EDM factor, s — spin, η ~ 10-15 — defines the proportionality between the EDM and the spin.

When considering the dynamics of the spin vector relative to the momentum, the difference between the spin precession rate and the orbital rotation rate is characterized by the spin tune, which takes the values νsBand νsEfor magnetic and electric fields, respectively:

νsB=γG,    (2)

νsE =γβ2G-1γ2-1=γG-G+1γ.   (3)

STRUCTURE WITH “FROZEN” SPIN

The frozen-spin (FS) method involves creating conditions for continuous alignment of the momentum vector and the horizontal projection of the spin vector as the particle circulates in the ring [4]. Measurement of the proton and deuteron EDM in the FS mode can be carried out within a single structure.

Frozen-spin structure for the proton

To implement the frozen-spin condition for the proton, a purely electrostatic structure is required at the “magic” experimental energy of E=233 MeV, which is lower than the energy E=270 MeVcorresponding to the maximum analyzing power of the carbon polarimeter for the proton [5]. Cylindrical electrostatic deflectors with a radial electric field gradient E(ρ)∼1/ρ, where ρis the distance from the central axis of the electrodes, are considered as bending elements. The total length of the bending elements is:

LE=2πmc2q∙γ2-1γE ,    (4)

where E  – is the electric field strength at the reference orbit.

As an estimate of the technically achievable upper limit of the electric field strength for cylindrical deflectors, a value of E=7.0 MV/mis adopted [6]. In this case, their effective total length for the proton will be L=375 m, as shown in Fig. 1.

An additional advantage of a purely electrostatic ring structure is the alignment of the deflector polarities for counter-rotating beams.

Frozen-spin structure for the deuteron

Due to the absence of a “magic” energy for the deuteron, its study in the FS mode, unlike the proton, requires the use of combined electric and magnetic fields, related by the following relation [7]:

By=Ex1-γ2-1GGcγγ2-1.    (5)

For the length of cylindrical elements with combined fields (or E+B elements), the following expression holds:

LE+B=2πmc2q∙Gβ2γ3EG+1=LE∙Gγ2G+1.    (6)

Transitioning to the measurement of the deuteron EDM at the maximum energy E=135 MeV/nucleon[8] in the previously considered electrostatic ring requires a reduction of the electric field magnitude and the addition of a uniform magnetic field.

Frozen-spin structure for both proton and deuteron

To measure the EDM of both particles within a single ring, cylindrical elements with combined fields are required, allowing independent adjustment of the electric and magnetic field strengths.

Figure 1 shows the dependence of the total length of the cylindrical elements on the maximum field strength for two different cases: a purely electrostatic ring for the proton and an electromagnetic ring for the deuteron. The difference in the signs of the electric field indicates the need to switch the polarity for different particle types. According to Fig. 1, the minimum length of cylindrical deflectors with a maximum field of E=7.0 MV/m, sufficient for measuring the deuteron EDM at the maximum energy E=135 MeV/nucleon, is L=96 m, corresponding to a relatively compact ring. According to Eq. 6, measuring the proton EDM in a compact structure requires lowering the experimental energy to E=75 MeV. This value differs from the “magic” energy, reduces the polarimeter efficiency, and necessitates the use of combined fields. The main parameters of frozen-spin structures are presented in Table 1.

In structures with combined fields, magnetic fields of opposite polarity must be applied in the particle channels, requiring their independent adjustment in both forward and reverse directions.

Determination of guiding fields in the frozen structure

The effect of the electric and magnetic fields of the E+B elements on the spin and orbital dynamics of the studied particles is different. Under the condition of orthogonality between the momentum vector and the external fields, the following relation holds for the ratio of the angular velocities of momentum rotation in the magnetic ΩpB and electric ΩpE fields:

λ=ΩpBΩpE=G+1Gγ2-1.    (7)

The ratio of the precession angular velocities due to the MDM in the magnetic ΩMDMB and electric ΩMDME fields:

ΛE+B=ΩMDMBΩMDME=λ∙γG+1γG+γγ+1β2=G+1γG(γ-1)(Gγ+1+1)-1 .    (8)

From Fig. A1, it follows that 0<∣λ∣<1: the electric field acts as the guiding field for the proton momentum in the considered energy range. From Fig. A2, in the case of the deuteron, ∣λ∣>1and λ<0: the momentum is guided by the magnetic field, opposite to the rotation in the electric field within the considered energy range.
From Fig. B1, at an energy of 75 MeV, ∣ΛE+B∣>1and ΛE+B>0, indicating that the magnetic field guides the spin vector of the proton. From Fig. B2, in the case of the deuteron, ∣ΛE+B∣≫1and ΛE+B<0: the spin-vector rotation is predominantly due to the magnetic field and is opposite to the rotation in the electric field.

Figures A1 and B1 show the sign change of λand ΛE+Bwhen crossing the magic energy Emag=233 MeV. Thus, for E>Emag, the rotation of both the momentum vector and the proton spin vector in the magnetic field becomes opposite to the rotation in the electric field.

Effect of deflector field nonlinearity

Due to betatron oscillations, particles oscillate around the reference orbit, which leads to the emergence of nonlinear effects from the electric field of the electrostatic deflectors [9]:

Ex=κρ+x=E01+xρ=E0∙(1-xρ+xρ2-xρ3+… ),    (9)

where κ is the electric rigidity, ρ is the curvature radius of the reference orbit, E0 is the reference field strength, and xis the radial displacement relative to the reference orbit (x≪ρ). The electric field of a deflector with a smaller curvature radius is more sensitive to radial particle deviations due to the increased contribution of higher-order terms in the field strength. The effect of the nonlinearity of the electrostatic deflector field is most significant for both the spin and orbital dynamics of particles in the case of a compact ring.

STRUCTURE WITH “QUASI-FROZEN” SPIN

The quasi-frozen spin (QFS) method involves the periodic restoration of the alignment between the spin vector and the momentum vector using compensating elements, allowing purely magnetic facilities to be adapted for experiments on the measurement of the EDM of charged particles [10]. Wien filters (WF) are the most optimal compensating elements for use in a quasi-frozen-spin structure [11].

Properties of a straight Wien filter

The design of a straight Wien filter involves creating crossed magnetic and electric fields. Adjusting the field strengths allows the Lorentz force to be canceled, preventing orbit distortion. The fabrication of straight electrodes is significantly simpler compared to cylindrical deflectors and allows achieving higher field strengths, up to E=13 MV/m. In addition, it avoids the orbital nonlinearities that arise in E+B elements with nonzero curvature in FS structures. The differing effects of the WF fields on spin-vector dynamics are determined by the ratio of the MDM precession angular velocities:

ΛWF=ΩMDMBΩMDME=γG+1-γG+γγ+1β2=γγG+1(1-γ)(Gγ+1+1) .    (10)

From Fig. 3, it follows that in the case of the Wien filter ∣ΛWF∣>1and ΛWF<0within the considered energy range: the magnetic field acts as the guiding field for compensating the spin-vector phase slip for both the proton and the deuteron, while the directions of spin-vector rotation in the magnetic and electric fields are opposite.

Effect of periodicity on the EDM signal

Exceeding the spin-vector rotation angle Фs  with longitudinal polarization relative to the momentum rotation angle Фp  in an external field by more than π2 leads to a loss of the accumulated EDM signal:

Фs = νs Фp ≤π2 .    (11)

The minimum number of periods in the QFS structure depends on the particle type and the experimental energy.

Nmin≥2πФp  ≥4γG.    (12)

Thus, the minimum structural periodicity for studying the proton EDM without signal loss is Nmin=8, which corresponds to an energy of Emin=114 MeV, whereas for the deuteron, the accumulated EDM signal is preserved for any number of structure periods.
The presence of spin-vector oscillations around the longitudinal axis in the QFS leads to a slowdown in the growth of the EDM signal compared to a frozen-spin structure, which is described by the expression:

J0=1-Фs 24=1-14∙2πγGN2,    (13)

where J0is the factor by which the maximum EDM signal amplitude is reduced, and Фs  is the maximum angle of deviation of the spin vector from the longitudinal direction.

Figure 4 shows that the periodicity of the QFS structure has a significant impact on the J0factor for the proton. Reducing the experimental energy brings the QFS structure closer to the FS configuration but leads to a substantial decrease in the polarimeter’s analyzing power. Increasing the number of periods to Nmax=16enhances the J0values but does not sufficiently approximate the structure to the ideal case. In the case of the deuteron, J0≈1for all N, and the QFS structure exhibits minimal deviation from the FS structure in terms of the EDM signal growth rate parameter. The main parameters of the quasi-frozen spin structures are presented in Table 2.

CONCLUSION

The structures considered in this work, both with frozen and quasi-frozen spin, allow for the study of the EDM of both the proton and the deuteron.

For the investigation of the proton EDM, the frozen-spin structure at the magic energy is of greatest interest. However, the length of such a setup becomes considerable and requires the use of cylindrical elements. For studying the deuteron EDM in a frozen-spin structure, both a magnetic field and the possibility of switching the polarity of the electric field are required. Investigation of the proton EDM at lower energy is feasible in a compact structure optimized for the measurement of the deuteron EDM at 270 MeV.

The quasi-frozen spin method can be applied in existing facilities. Spin compensation is performed sequentially using a Wien filter. Unlike cylindrical deflectors, the Wien filter is relatively simple to manufacture and does not introduce nonlinearities in the electrostatic field. The efficiency of EDM signal accumulation depends on the periodicity of the structure, which is most significant in the case of the proton. Nevertheless, the quasi-frozen spin method allows the development of the EDM measurement technique for the proton as well.

FUNDING

The study was supported by a Russian Science Foundation grant № 25-72-30005, https://rscf.ru/project/25-72-30005/.

CONFLICT OF INTEREST

The authors declare that they have no conflicts of interest.

REFERENCES

- Sakharov A. D. Violation of CP invariance, C asymmetry, and baryon asymmetry of the Universe // JETP Lett. — 1967. — Vol. 5, № 1. — P. 24–27 — DOI: 10.1070/PU1991v034n05ABEH002497

- Khriplovich I.B., Lamoreaux S.K. CP Violation Without Strangeness: Electric Dipole Moments of Particles, Atoms, and Molecules. — Berlin: Springer, 1997.— DOI: 10.1007/978-3-642-60838-4.

- Bargmann V., Michel L., Telegdi V.L. Precession of the polarization of particles moving in a homogeneous electromagnetic field // Phys. Rev. Lett. — 1959. — Vol. 2. — P. 435. — DOI: 10.1103/PhysRevLett.2.435

- Anastassopoulos V., Babusci D., Bachtis D. et al. A Storage Ring Experiment to Detect a Proton Electric Dipole Moment // Rev. Sci. Instrum. — 2016. — Vol. 87. — P. 115116. — DOI: 10.1063/1.4967465.

- McNaughton M.W. et al. The p–C analyzing power between 100 MeV and 750 MeV // Nucl. Instrum. Methods Phys. Res. A — 1985. — Vol. 241. — P. 435–440. — DOI: 10.1016/0168-9002(85)90595-9

- Abusaif F., et al. (CPEDM Collaboration). Storage ring to search for electric dipole moments of charged particles: Feasibility study // CERN Yellow Reports: Monographs. — 2021. — Vol. 3. — P. 246. — CERN-2021-003. — DOI: 10.23731/CYRM-2021-003.

- Anastassopoulos D., Anastassopoulos V., Babusci D. et al. AGS proposal: Search for a permanent electric dipole moment of the deuteron nucleus at the 10^–29 e·cm level. — Tech. Rep. BNL, 2008 DOI: none.

- Müller F., et al. Measurement of deuteron–carbon vector analyzing powers in the kinetic energy range 170–380 MeV // Eur. Phys. J. A. — 2020. — Vol. 58. — P. 211. — DOI: 10.1140/epja/s10050-020-00215

- Valetov E., Berz M. Derivation of Analytic Formulas for Electrostatic Deflector Aberrations, and Comparison with the Code COSY INFINITY // MSU Report MSUHEP 180212 (Michigan State University, East Lansing, MI) — February 12, 2018.

- Senichev Y., Lehrach A., Lorentz B., Maier R., Andrianov S., Ivanov A., Chekmenev S., Berz M., Valetov E. Quasi-frozen spin method for EDM deuteron search // Proc. of IPAC 2015 (Richmond, VA, USA). — 2015. — P. 213. — DOI: 10.18429/JACoW-IPAC2015-MOPWA044

- Kolokolchikov S., Senichev Y., Aksentyev A., Melnikov A., Palamarchuk P. Quasi-frozen spin for both deuteron and proton beam at periodic EDM storage ring lattice

TABLES

Table 1. Main Parameters of Frozen-Spin Structures

| Structure Parameters | FS |  | FScmp |  |
| --- | --- | --- | --- | --- |
|  | p | d | p | d |
| E, MeV | 233 | 270 | 75 | 270 |
| γ | 1.250 | 1.144 | 1.08 | 1.144 |
| EE+B, MV/m | 7 | -1.78 | 7 | -7 |
| BE+B, T | 0 | 0.069 | 0.020 | 0.274 |
| E + B Length, m | 375 |  | 95 |  |

Table 2. Main Parameters of Quasi-Frozen-Spin Structures

| Structure Parameters |  | QFS |  | QFScmp |  |
| --- | --- | --- | --- | --- | --- |
|  |  | p | d | p | d |
| E, MeV |  | 270 |  | 73 | 270 |
| γ |  | 1.289 | 1.144 | 1.078 | 1.143 |
| EWF , MV/m |  | 13 | -2.68 | 13 | -13 |
| BWF, T |  | -0.068 | 0.014 | -0.068 | 0.068 |
| WF Length, m |  | 250 |  | 52 |  |
| J0 | N = 8 | 0.179 | 0.996 | 0.433 | 0.996 |
|  | N = 16 | 0.795 | 0.999 | 0.858 | 0.999 |

FIGURE CAPTIONS

Fig. 1. Dependence of the length of cylindrical deflectors in a frozen-spin structure on the electric field strength at the maximum experimental energy (blue line – electrostatic ring; purple line – electromagnetic ring).

Fig. 2. Comparison of angular velocities in the magnetic and electric field of an E+B element: (a) orbital rotation, (b) spin precession (blue line – proton; purple line – deuteron).

Fig. 3. Comparison of spin precession angular velocity in the magnetic and electric field of the Wien filter for different particle types (blue line – proton; purple line – deuteron).

Fig. 4. Dependence of the EDM signal growth factor on the experimental energy for different structure periodicities (N = 8: proton – light blue line, deuteron – gray line; N = 16: proton – blue line, deuteron – purple line).

FIGURES

Fig. 1.

Fig. 2.

Fig. 3.

Fig. 4.