FEATURES OF THE MAGNETO-OPTICAL STRUCTURE OF
A MULTIFUNCTIONAL FIGURE-8 RING

Kolokolchikov S.D.1,2, Aksentyev A.E.1,2,3, Melnikov, A.A.1,2,4, Senichev Yu.V.1,2

1Institute for Nuclear Research RAS, Moscow, Russia,

2Moscow Institute of Physics and Technology, Dolgoprudny, Russia,

3Moscow Engineering Physics Institute, Moscow, Russia,

4Institute for Theoretical Physics L.D. Landau, Chernogolovka, Russia

The article presents a fundamental approach to figure-eight ring structure for multiple physical programs. Firstly, the unique shape of the ring resolves the issue of spin resonance crossings during the acceleration of a polarized beam. Secondly, this structure can be used for Electric Dipole Moment (EDM) measuring experiments and axion search. Thirdly, the structure can be adapted for heavy-ion beam acceleration.

The acceleration of a polarized beam is of great interest for collider experiments. The issue of crossing various types of spin resonances arises, leading to beam depolarization. Using a figure-8 shaped ring, one arc compensates for the spin precession caused by the Magnetic Dipole Moment (MDM) induced by the other. Thus, the spin tune is zero, and the ring becomes spin-transparent (ST-mode) [1].

For polarized proton beam, crossing the transition energy represents a significant challenge. In order to address this issue, the technique of the dispersion function variation is employed [2]. Thus, the transition energy rises above the experimental energy or even reaches a complex value.

The deuteron electric dipole moment (EDM) can be measured at low energies using the quasi-frozen spin (QFS) method in a figure-8 shape lattice configuration [3]. To accumulate the EDM signal, both solenoids and Wien filters must be installed in each straight section. The lattice design features three independent sextupole families in the bending arcs to achieve spin coherence of polarized beams. The figure-8 shaped storage ring configuration provides enhanced sensitivity as a broadband axion antenna, particularly for studying g-2 precession in low-frequency regimes [4].

The lattice can be utilized for heavy-ion beam acceleration. In this case, the main requirement is to achieve a sufficient beam lifetime. This requires compensating intrabeam scattering (IBS) effects through combined electron and stochastic cooling, which can be efficiently achieved in a lattice with minimally modulated Twiss functions.

Acknowledgments

The study was supported by a Russian Science Foundation grant № 25-72-30005,

https://rscf.ru/project/25-72-30005/

References

- Ya. S. Derbenev et al. Polarization Preservation and Control in a Figure-8 Ring, Int.J.Mod.Phys.Conf.Ser. 40 (2016) 01, 166009 DOI: 10.1142/S2010194516600909

- Senichev, Y.V., Chechenin, A.N. Theory of “Resonant” lattices for synchrotrons with negative momentum compaction factor. J. Exp. Theor. Phys. 105, 988–997 (2007). DOI: 10.1134/S1063776107110118

- Yu.Senichev et al., in ICAP2015 Proccedings, Shanghai, 2016, ed. by C.Kwok, R.W.Volker, T.Chuanxiang, Y.Heping

- N.N.Nikolaev, Spin of protons in NICA and PTR storage rings as an axion antenna. Jetp Lett. 115, 639–643 (2022). DOI: 10.1134/S0021364022600653